[See end of this file for revision history]

A FIX FOR PROBLEMS WITH PATCHX.CPE

**********************************

Many developers have reported problems with PATCHX. There were many

reported symptoms but the most common was that certain parts of the

new kernel did not seem to function correctly, particularly if

another program was run immediately after PATCHX.

A close investigation of the PATCHX.CPE program showed that:-

1) It has an obsolete MW3 start-up!

2) It has cache coherency problems: This is because it runs

from cached memory and copies to cached memory yet it does not

flush the cache before executing the new kernel. This means

that if any of the old kernel remains in the cache when the

replacement is executed then the cpu will execute a mixture of

the old and new kernel code!

We have isolated the new kernel data from PATCHX.CPE and incorporated

them into the NEWDEX installation program along with the original

debugger stub update (necessary for debugging with callbacks and for

hardware data-breakpoint support).

The result is SNPATCH.CPE which installs much faster than PATCHX and

does not suffer from the aforementioned cache coherency problems.

Note that SNPATCH replaces both PATCHX *and* NEWDEX. You would be

advised to delete those latter programs from your PC.

So, for those of you setting up the PSX in a batch file...

change this:                    to this:-

RESETPS 1                       RESETPS 1

RUN /w4 PATCHX                  RUN /w4 SNPATCH

DELAY

RUN /w4 NEWDEX

...and the result is a lot faster and more reliable.

NOTE: The kernel installed by SNPATCH is the same as the one in

PATCHX, We just install it correctly. Unfortunately the kernel itself

is a little unstable for a short while (less than 0.5 sec) so if you

want to RUN something else in the same batch file that runs SNPATCH

then you would be advised to insert a small (0.5 sec) delay before

you RUN your app. There is a suitable DELAY program in this archive,

(execute DELAY /? for help):-

RESETPS 1

RUN /w4 SNPATCH

DELAY

RUN /w4 MYPROG

-----------------------------------------------------------------

WHILE WE HAVE YOUR ATTENTION: *** IMPORTANT WARNING ***

WE KNOW THAT SOME OF YOU ARE USING OBSOLETE MW3 VERSIONS OF LIBSN

You may have updated LIBSN at an earlier date and then overwritten it

with an older LIBSN from a recent library update. PLEASE CHECK NOW!

The current LIBSN.LIB is about 19 Kbytes in size and contains C++ support

and user-definable ramsize support. Do not worry about the

larger size - the extra modules are only linked if referenced.

To check that you are using the correct LIBSN change to your GNU

library directory and enter:-

PSYLIB /l libsn.lib

The resulting output should include the PUREV module near the top

(this is the __pure_virtual function support required by GNU C++ from

version 2.6.3 on). If you have this module then you can be fairly

sure you have an up-to-date LIBSN.LIB

If you do not have this module in your lIBSN.LIB then you should

stop what you are doing and update to the latest LIBSN immediately.

If you continue to use an old MW3 LIBSN you *will* have problems

because the MW3 start-up read dip switches to determine ram size at

startup. Since the ISA card PSX does not have these DIP switches it

will not correctly initialise the stack and heap!

ALSO NOTE that you should not use 8M.OBJ and 2M.OBJ to adjust

your program for DTL-H2000 or production Playstion.

Use the variables referenced by LIBSN.LIB start-up (e.g. _ramsize ).

See the readme with LIBSN for further details.

--------------------------------------------------------------------

*** ANOTHER WORD OF WARNING ***

Although SN Systems are not aware of the mechanism used by many of

the kernel functions, in light of the PATCHX problems we would advise

any developers to beware of other cache coherency problems in their

own code.

The only similar situation which comes to mind would apply to anyone

using code overlays on the Playstation. You should be aware than any

data read or copied into memory is not placed into the instruction

cache. Therefore, if this data contains program code to be executed you

should force the cpu to flush the instruction cache before calling

the new program code.

Source code for SNPATCH is available for your reference. Although

it is written in assembly language you can very easily assemble

fragments using ASMPSX to produce object modules for linking to C.

--------------------------------------------------------------------

REVISION HISTORY

After running SNPATCH you will find that RUN (by itself) will report

a higher version number (currently 4.04)...

***07-Mar-95

SNPATCH with debug stub 4.03 fixes a problem in 4.02 whereby a

pollhost() call in a callback (e.g. VBL) causes fileserver operations

(or printf() ) in your main program to fail (or even hang).

*** 31-May-95

SNPATCH 4.04:

A minor update on SNPATCH 4.03, this just adds one more handy break

function to the debug stub:-

BREAK $0407

...which pauses program execution at that point. Execution can be

resumed by hitting f9 (RUN) in the debugger.

Remember that this will only function if the debug stub is hooked in.

i.e. it won't work if you code is booted of CD using a RESETPS 0

This has also been added to LIBSN.H as a macro for use from C:-

#define	PSYQpause() asm("break 1031")	/* inline to keep variable scope */

Please copy the enclosed LIBSN.H file to your Playstation GNU include

directory.

To use this in your C source add a line like:-

PSYQpause();

Note that the C local variable scope at the pause point is preserved

if you are debugging.

*** 23-Aug-95

SN versions of SCE's PATCHJ.CPE and PATCHW.CPE

Note: PATCHJ is identical to the old PATCHX and it's equivalent is

the old SNPATCH. For consistency I have renamed this to SNPATCHJ.CPE

SNPATCHW.CPE is our fixed version of SCE's PATCHW.CPE

Here is the data from SCE:-

>Updates of patchx.cpe:

>===================================================================

>

>In addition to the existing KANJI fonts, the user-defined character

>font (245 characters) is added to PlayStation for overseas as the

>built-in font data.

>

>       * Latin alphabets with diacritical marks

>       * Greek alphabets with diacritical marks

>       * Kyrill alphabets

>       * Latin alphabets

>

>     We release patchw.cpe for these data. And the name, patchx.cpe,

>is changed to patchj.cpe.  Patchw.cpe can be treated in the same way

>as patchj.cpe.

>

> patchj.cpe : for SCEI's titles

> patchw.cpe : for SCEA's or SCEE's titles

>

>     Use Krom2RawAdd2() instead of Krom2RawAdd() when the access to

>the user-defined character data is made after the executing of

>patchw.cpe.  Krom2RawAdd2() is included in libgpu.lib of PS-X

Library

>Version 3.2 or later.

>

>     Refer to 'Updates of KANJI Font Utility' for details (Patchy.cpe

>which was included in this package is the same file as patchw.cpe).

>

-- Andy Beveridge